Activity 1

DC Motors

Student Name:		ZHI CHEN	Student No: 20144855

Part 1.

Complete lesson 21 from the Elegoo Super Starter kit.

Part 2

# Hardware Required

- Arduino Uno R3 Board

- 2 X DC Motors (Kit)

- L293D motor driver board

- 12V Plugpack

- hook-up wires

- breadboard

# Overview

How to control speed and direction of 2 DC motors using the L298N motor driver board with Arduino Uno R3

We will use the 12V Plugpack to provide required power as the Arduino cannot supply enough voltage/current.

# Wiring

+5V – RED

GND – Black

ENA – Arduino pin 9

ENB – Arduino pin 3

L298N Board

Motor 1 – Output 1 & 2

Motor 2 – Output 3 & 5

IN1 - Arduino pin 8

IN2- Arduino pin 7

IN3 – Arduino pin 5

IN4 - Arduino pin 4

**Leave regulator enable jumper in place as motors supply is less than 12V**

Program code is available on Blackboard under Assessments/Activity 1

- Insert picture of your working DC Motor Lab Part 2 here, must include your name!

- The most common actuator in mobile robotics (in general) is the ……. Motor

- AC

- DC

- Three phase

- Solenoid

- The circuit below is called a ……………… and will give direction control of a ………… motor.

- T bridge, DC

- T bridge, AC

- H bridge, DC

- H bridge, AC

- Write a for loop count from 0 to 50.

for (int i = 0; i <= 50; i++) {

Serial.print("Current Count: ");

Serial.println(i);

delay(100);

}

- Flemings left hand rule uses conventional current flow.

- Name 6 parts of a DC motor.

    - Stator

    - Field winding

    - Armature

    - Brushes

    - Commutator

    - Shaft

- Why is a permanent magnet so called?

Because it retains its magnetic properties without needing an external power source.

- In a typical 3 coil DC motor how far apart are the coils in degrees?

120 degree, as a full circle is 360 degree, 3 coils are spaced at 360/3 =120 degrees.

END

Please submit completed worksheet in blackboard